|  | ASSESSMENT OF A NEXT GENERATION GRAVITY MISSION FOR MONITORING THE VARIATIONS OF EARTH’S GRAVITY FIELD Issue 1.0 WP3200 Scientific Assessment of the Baseline Mission |  |
| --- | --- | --- |

| Written by | Responsibility |
| --- | --- |
| T. van Dam | Author |
| P.N.A.M. Visser | Author |
| Verified by |  |
| Approved by |  |
| Documentation Manager |  |
| P.N.A.M. Visser |  |

CHANGE  RECORDS

| ISSUE | DATE | § CHANGE RECORDS | AUTHOR |
| --- | --- | --- | --- |
| 1.0 | November 2010 |  | TD,PV |

TABLE OF CONTENTS

1.	INTRODUCTION	4

1.1	Scope and Purpose	4

2.	Documents	4

2.1	Applicable Documents	4

2.2	ESA Reference Documents	4

2.3	Further Reference Documents	4

3.	Selected GRAVITY FIELD MISSION SCENARIOS RUN BY the End-to-end simulator	5

4.	MISSION PERFORMANCE AND SCIENCE REQUIREMENTS	5

5.	Conclusions and recommendations	5

6.	ACRONYMS	6

# INTRODUCTION

## Scope and Purpose

This document is submitted in fulfillment of WP 3200 of the Next Generation Gravity Mission (NGGM) study. This document includes a summary of:

- Selection of cases run by the End-to-End simulator (Chapter 3);

- Comparison between performance for these cases with the science requirements (Chapter 4);

- Conclusions and recommendations (Chapter 5).

# Documents

## Applicable Documents

- Assessment of a Next Generation Gravity Mission to monitor the variations of Earth’s gravity field, Statement of Work, EOP-SF/2008-09-1334, Issue 2, 20 November 2008, Appendix 1 to AO/1-5914/09/NL/CT

- Special Conditions of Tender, Appendix 3 to AO/1-5914/09/NL/CT

- Draft Contract. Appendix 2 to AO/1-5914/09/NL/CT.

## ESA Reference Documents

- T. van Dam et al., Monitoring and Modelling Individual Sources of Mass Distribution and Transport in the Earth System by Means of Satellites, Final Report, ESA Contract No. 20403, November 2008

## Further Reference Documents

- Study Technical Note “Requirement Analysis”, NGGM_SCI_1, Issue 1, Revision 1, 08-February, 2010

- WP1100 Requirements Analysis, 4 December 2009

- TN3_IAPG: From Science to Sensor requirements, TN3: Observing Techniques and Instrument Concepts, 29 October 2010 (WP2110)

- TN_IAPG: Reprocessing of mass transport data, 29 October 2010 (WP2320)

- WP2330 Backward Module, 1 November 2010

- WP2420 Mission Architecture Definition/Supervision, 2 November 2010

# Selected GRAVITY FIELD MISSION SCENARIOS RUN BY the End-to-end simulator

- A total of 6 different tandems were investigated in detail by the end-to-end simulator. These tandems were selected on the basis of analyses conducted with the quick look tools [RD-7].

Gravity field retrieval experiments have been conducted for a number of mission scenarios, including the single-tandem in-line polar, in-line SSO, pendulum polar and cartwheel formations, and the dual-tandem Bender constellation (63o and 900 inclination). First, ll-SST only solutions were produced and analyzed in detail. The parameterization was established based on a limited number of tests, which indicated that the accelerometer sensor noise is by far the dominant sensor error source. It was found that for the in-line polar formation the impact of this error source could be mitigated by estimating hourly piecewise linear empirical accelerations and this parameterization was adopted for the cases where all sensor errors were included.

The first series of gravity field retrievals (so-called Case 1) were based on the assumption that all sensors are perfect and that the residual gravity field consists of hydrological gravity field changes only. In principle, retrieval errors would then be caused by temporal aliasing only, i.e. hydrological gravity field changes WITHIN the retrieval period. The retrieval period was taken equal to a full repeat period of 32 days, or equal to (multiples) of sub-cycles: 4, 7 and 14 days. It was found that the impact of temporal aliasing on the retrieved gravity field is already distinctly different for the different mission scenarios. The pendulum and cartwheel formations seem to be affected most, especially at the low spherical harmonic (SH) degrees. For the shortest sub-cycle of 4 days, the cartwheel formation has relatively good performance, whereas the pendulum formation has a very bad performance (especially for the very low degree zonal coefficients).

When all sensor errors are included and also more residual temporal gravity field signal is present (i.e. 10% of the atmospheric and ocean mass changes), the relative performance for the different mission scenarios changes from smaller to greater extent. For these retrievals, the piecewise linear empirical accelerations were estimated simultaneously with the gravity field parameters. It was found that this co-estimation was not only very efficient in absorbing the effect of sensor errors, but most likely also alters the impact of temporal aliasing. For example, the (relative) performance for the pendulum formation improves significantly for the 7-day sub-cycles and for the 7- and 32-day periods. The dual-tandem Bender constellation displays a good performance for all periods, especially for the 4-day sub-cycle (which can be explained by the much better and denser global coverage with two tandems for such a short period), but is now in fact outperformed significantly by the pendulum formation for SH degrees larger than about 20 for the full 32-day repeat period (this is to a lesser extent also the case for the 7-day sub-cycle and the 14-day period). In general, the pendulum and cartwheel formations seem to suffer relatively much from the sensor errors, especially considering the gravity field retrieval results at the very low SH degrees (<10).

When analyzing gravity field retrieval errors in the spatial domain (e.g. geoid errors as a function of geographical latitude and longitude), the error patterns for the Bender dual-tandem and pendulum configurations display less pronounced stripes than for the in-line polar, in-line SSO and cartwheel polar formations.

All the obtained gravity field retrieval performance figures for the ll-SST computations are corroborated by formal error propagations of the covariance matrices that are obtained as by-product of the least-squares estimation process. Based on formal errors and covariance functions, it can be anticipated that the Bender dual-tandem and pendulum polar configurations will lead to gravity field solutions with a more homogeneous and isotropic quality. Furthermore, the Bender and pendulum polar scenarios demonstrate some synergy potential: the Bender constellation delivers the best results at low degrees, whereas the pendulum formation results in the lowest noise level at high degrees. It can be advised to put efforts into the design of a new formation type where the strong features of the Bender and pendulum configurations are combined. For instance, one or both in-line pairs that form the Bender constellation can be replaced with the pendulum pair(s).

As stated before, the same parameterization (i.e. estimated parameters) was adopted for all ll-SST mission scenarios. It is fair to assume that for different mission scenarios, different parameter sets lead to optimal results. For example, the performance for the cartwheel and pendulum formations is relatively bad for the very low degrees. This might be due to the higher impact of sensor errors (e.g. attitude errors will lead to larger observation errors because the ll-SST signal is simply much bigger, especially at the very low frequencies), but also a higher frequency of estimated empirical accelerations might help (although this will reduce the observability of gravity parameters and thus a trade-off has to be made).

A rigorous assessment has been made of the possible added value of Satellite Gravity Gradient (SGG) observations. The general conclusion is that the SGG noise level makes it challenging to improve the ll-SST based gravity field solutions: this noise level has to be reduced significantly compared to the currently achievable level. For a monthly gravity field recovery, the added value of SGG data becomes visible only if the noise level is reduced by one order of magnitude, if an in-line polar/SSO formation is considered. For other configurations (Bender-polar, pendulum, cartwheel, or a combination), a reduction of noise level by two or even more orders of magnitude is needed. Importantly, this concerns both accelerometer sensor noise and attitude reconstruction noise.

Still, it would be pre-mature to conclude that an on-board gradiometer with a currently achievable noise level is useless in mass transport monitoring. First of all, all the data combination attempts made so far were based on the assumption that mass transport is described by the formula “HIS-0.1AO”, where significant variations within the given month are associated with the second term (“-0.1AO”). Therefore, temporal aliasing, which is a major source of errors in ll-SST-only models, is largely dependent on the assumptions about the second term. A comparison with models based on real GRACE data allows us to suggest that our assumption is rather optimistic: the errors in GRACE models exceed those observed in the “In-line polar” scenario by about one order of magnitude. Even though there are on-going research activities aimed at an improvement of GRACE data processing strategies, it is still not guaranteed that a 10-times reduction of temporal aliasing will be achieved by the moment when the mission under consideration becomes operational. On the other hand, noise in SSG data, as was shown above, is mostly dictated by the sensor performance and will stay practically the same even if the effects of temporal aliasing are 3-10 times higher than assumed. This means that the ratio between noise in SGG and in SST data might be 3-10 smaller than assumed in the “nominal noise” scenario. In other words, an assumption that noise in SGG data is reduced 3 or even 10 times might be closer to the reality when the added value of SGG data is discussed. This leads us to the conclusion that it is still worthwhile to derive SGG observations from on-board accelerometers in the context of monthly gravity field retrievals, if an in-line polar (or an in-line SSO) formation is chosen to collect gravity field measurements.

Secondly, the currently achievable noise level in SGG data still allows these data to provide a valuable contribution in the context of gravity field recovery for short time intervals. For a 4-day interval and for estimating the gravity field complete to degree 80, the added value is very significant when a single-pair formation is considered (though the Bender constellation does not benefit from SGG data because two pairs of satellites in that case still ensure a sufficiently dense spatial coverage). However, it is certain that for shorter time intervals (2 or even 1 days) or for a gravity field recovery up to a high spherical harmonic degrees (e.g. a degree above 112 for a 7-day interval and a single-pair scenario), the spatial coverage will be insufficient, and then the presence of SGG observations will be essential. It is worth adding that a gravity field recovery on a 1- or 2-day basis up to a high degree might be needed not only when rapid mass transport processes are the final target. One should keep in mind that models of rapid mass transport processes are also used for temporal de-aliasing of satellite gravity data. An elimination of rapid mass transport signals is critical for reducing noise level in the data and, therefore, for achieving the highest spatial resolution at all time scales (including even secular variations caused, e.g., by melting of polar ice sheets). Currently, models of rapid mass transport model are hardly based on satellite gravity data,whereas the accuracy of these models is limited. As it is already mentioned above, this is one of the major sources of errors in GRACE-based model. Ability of future satellite gravity missions to improve rapid mass transport models will be, with no doubt, a highly desirable feature in the context of all potential applications of satellite gravimetry.

# MISSION PERFORMANCE AND SCIENCE REQUIREMENTS

Mission design is driven by well-defined scientific targets.   A mixture of requirements of spatial resolution, temporal behaviour (both resolution and mission duration) and geoid/gravity precision can be accomplished by proper orbit design, choice of observation type and hardware performance.

In this document, we have thoroughly reviewed the science that has been revealed through TVG.  We have additionally, quantified mass transport signals in terms of spatial and temporal scale, which could be analyzed with an NGGM having requirements surpassing what exists today.

In prioritizing the mass transport processes to be observed by an NGGM, it is important to take into account the following questions:

- Is the signal of sufficient magnitude to be observed by satellite gravity?

- Are there alternative more cost-effective techniques for observing the signal?

- How important is an improved quantification of a particular mass transport process for managing our environment and enhancing our ability to derive sustainable benefit from it?

With regards to Question 1, this document has identified numerous signals that are sufficiently large to be observed using satellite gravity.  An improvement in the temporal and spatial resolution and/or the accuracy of an NGGM with respect to present capabilities will increase significantly the number of mass transport signals, which can be studied.  In addition, these improvements may reveal, signals and processes, which are as yet unknown.

With regards to Question 2, we note that, currently only remote sensing techniques can be typically considered as a fair alternative to satellite gravimetry. In-situ observations usually lack spatial coverage, so that the quality of derived models may show large spatial variations.  However, remote sensing is only sensitive to surface or near-surface processes, while many of the mass transport processes discussed in this document may not have an expression at the surface.  In addition, certain quantities such as mass balance of the ice sheets or groundwater depletion, which require observations of mass change, cannot be measured using remote sensing techniques.

Throughout this document, we have attempted to justify our desires for improved quantifications of the various mass transports in terms of their contribution to understanding the interaction of the various components of the Earth system.  While not explicitly stated, an understanding of the cause-effect relationship between human activities and environmental change, when it exists, is sought.  In addition, certain mass-transport processes, including earthquakes, volcanoes, and GIA, which are not the result of human activities, nonetheless can have a dramatic effect on mankind.  Improving our understanding of these processes is also important for assessing the threat of natural hazards.

As described in [RD-4], the science requirements were translated to sensor and gravity field mission performance requirements.

|  | Wavelength | 10000 km | 1000 km | 200 km | 100 km |  | 10 km |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SH degree | 2 | 20 | 100 | 200 |  | 2000 |  |  |
| CGE | 10 mm |  |  | 3 |  |  |  |  |  |
|  | 1 mm | 2 |  |  | 4 |  |  |  |  |
|  | 0.1 mm |  | 1 |  |  |  |  |  |  |

Table 1 Requirements in terms of max. CGE for monthly solutions (The numbers in the grey boxes correspond to the first column of Table 2), the shaded areas on the right mark the restriction to more realistic values in case of a monthly satellite gravity field.

# Conclusions and recommendations

# ACRONYMS

AD				Applicable Document

BOL		Beginning of Life

BSM			Beam Steering Mechanism

C/C			Carbon-Carbon (composite)

CHAMP		CHAllenging Minisatellite Payload

COM			Centre of Mass

E2E			End-to-End

EOL			End of Life

FF				Formation Flying

GNSS			Global Navigation Satellite System

GOCE			Gravity field and steady-state Ocean Circulation Explorer

GPS			Global Positioning System

GRACE		Gravity Recovery And Climate Experiment

INRIM			Istituto Nazionale di Ricerca Metrologica

ITT				Invitation To Tender

KBR			K-Band Ranging

LEO			Low Earth Orbit

ll-SST			low-low Satellite-to-Satellite Tracking

LORF			Local Orbital Reference Frame

LRR			Laser Retro Reflector

MBW			Measurement Bandwidth

MST			Mission Simulation Tool

NGGM			Next-Generation Gravity Mission

P/L				Payload

POD			Precise Orbit Determination

PSD			Power Spectral Density

RD				Reference Document

RF				Radio Frequency

RMS			Root Mean Square

S/C			Spacecraft

SLR			Satellite Laser Ranging

SQUID			Superconducting Quantum Interference Device

SSO			Sun Synchronous Orbit

SST			Satellite to Satellite Tracking

TAS-I			Thales Alenia Space Italia

TBC			To Be Confirmed

TBD			To Be Defined

END OF DOCUMENT\*